AgileLight -800A Series

Satellite Fiber Optic Gyroscope

V1.0

Technical Manual

| Introduction |  |
| --- | --- |

AgileLight - 800A fiber optic gyroscope is designed specifically for space applications. It not only possesses core characteristics suitable for space environments, such as radiation resistance, low power consumption, long lifespan, vacuum environment tolerance, and rapid start-up response, but its key technical indicators, such as zero-bias stability, far surpass those of MEMS-type gyroscopes. With these advantages, this gyroscope can fully meet the attitude control requirements of various space missions, including communication, remote sensing, observation, and navigation, as well as at different orbital altitudes, effectively improving the platform's attitude control reliability.

This product adopts a mature satellite gyroscope design and has passed tests for ground vibration, shock, thermal vacuum, thermal cycling, total dose, and electromagnetic compatibility.

| - Commercial Spaceflight - Motion posture control - Damping of high-speed train swaying - Antenna/Radar/Optical Platform Stabilization - Monitoring of building structural deformation - Inertial search for north Applications | - Automated Cargo Handling Vehicle - Robot balancing - Guidance and navigation - Locking on the aiming system - oil drilling - Small inertial navigation system |
| --- | --- |

Performance indicators

| Fiber optic gyroscope specifications* | Measuring range |  | ±100°/s |
| --- | --- | --- | --- |
|  | Isothermal zero-bias stability (100 s, 1σ) |  | ≤0.02°/h |
|  | Zero bias across the entire temperature range (-20℃ to +50℃) |  | ≤±0.3°/h |
|  | Zero-biased repeatability |  | ≤0.04°/h (1σ) |
|  | Scale factor Nonlinearity |  | ≤200ppm |
|  | Scale factor repeatability |  | ≤200ppm |
|  | Angular velocity measurement error |  | Absolute error ≤5×10-4 °/s (Input ≤0.6°/s) |
|  |  |  | Relative error ≤ 5×10-4 °/s (0.6°/s ≤ Input ≤ 300°/s) |
|  | Random walk coefficient |  | ≤0.0003°/h 1/2 |
| physical indicators | volume |  | 90mm×90mm×66.5mm |
|  | weight |  | 450g±20g |
|  | Power consumption |  | ≤4.5w |
|  | Start-up instantaneous power |  | ≤12.5w (duration ≤10ms) |
|  | Operating temperature |  | -20℃~+50℃ |
|  | Storage temperature |  | -40℃～+60℃ |
|  | Operating relative humidity range |  | ≤70% |
|  | Storage relative humidity range |  | ≤70% |
|  | Communication interface |  | RS422 interface (dual-channel) |
|  | Power supply voltage |  | +5.2V±0.3V |
| Other technical indicators | Startup and signal stabilization time |  | ≤1min |
|  | Three-axis assembly angle correction data accuracy |  | ≤2′ |
|  | Remnant magnetic moment |  | ≤0.5Am 2 |
|  | Insulation resistance |  | > 10 MΩ |

Fiber Optic Gyroscope Specifications: Within the operating temperature range, temperature compensation can be applied—using the unit's internal temperature data and the correction formula derived from calibration tests—if necessary, to ensure the aforementioned performance specifications are met.

Interface definition

    - Mechanical interface :

- Structural envelope dimensions: (90±0.2) × (90±0.2) × (66.5±0.2) mm;

- Number and size of mounting holes: 4×Φ(4.5±0.1) mm;

- Mounting hole spacing: (75±0.1) mm×(75±0.1) mm;

- Mounting surface flatness: ≤0.1 mm/(100 mm×100 mm);

- Mounting surface roughness: ≤3.2μm;

- Mounting ear thickness: (3.5±0.1) mm;

- Product weight: 0.45 kg ± 0.02 kg;

- Mounting surface contact area: ≥7000mm2.

    - Power supply interface:

Input and output signals are transmitted through a J30JA-25ZKP type socket. The socket definition is shown in Table 1.

- Power supply voltage: +5.2V±0.3V;

- Ripple voltage: ≤150mV (PP);

- Ripple frequency: ≤10MHz;

- Surge current: ≤2.5A/10ms;

- Steady-state power consumption (full temperature range): ≤5W;

Start-up transient power consumption: ≤12.5W.

| Node number | Signal function description | Voltage V | Current A | Polarity | Notes (Shielding/Twisted Pair) |
| --- | --- | --- | --- | --- | --- |
| 1,14 | +5V power supply | Supply voltage +5 | <2 | + | The main line and return line are twisted together. |
| 3,16 | +5V power return line | 0 | <2 | - |  |
| 5,18 | The instruction receives a negative (RX-) signal. | RS422 standard | <0.1 | - | 5, 6 twisted pairs 18, 19 twisted pair |
| 6,19 | Command received positive (RX+) | RS422 standard | <0.1 | + |  |
| 7,20 | Negative data transmission (TX-) | RS422 standard | <0.1 | - | 7, 8 twisted pairs 20, 21 twisted pair |
| 8,21 | Data transmission in progress (TX+) | RS422 standard | <0.1 | + |  |
| 9,22 | Payload transmission positive (TX+) | RS422 standard | <0.1 | + | 9, 10 twisted pair 22, 23 twisted pair |
| 10,23 | Payload transmission negative (TX-) | RS422 standard | <0.1 | - |  |
| 12,24 | signal ground | / | / | / |  |
| 13,25 | chassis ground | / | / | / |  |
| 2,4,11,15,17 | null | / | / | / |  |

Product Dimensions

Product dimensions: 90×90×48.5 (mm)

Statement

In the event of a functional failure, please do not attempt to open the housing yourself. The product must be returned to the company for repair. Please note that any damage resulting from misuse or improper repair attempts is excluded from the company's warranty coverage.。

AgileLight -800A Series

Satellite Fiber Optic Gyroscope

Wuxi Bewis Sensing Technology LLC

Address: Building 30, No. 58 Xiuxi Road, Binhu District, Wuxi City, Jiangsu Province, China

Tel：+86 18921292620

Email:  sales@bwsensing.com Web:  www.bwsensing.com