V1.0

Technical Manual

BW-DC13P series

Thermal Imaging Dual Spectrum Camera

Introduction

Feature

BW-DC series is a self-developed intelligent gain control algorithm, detail hybrid enhancement algorithm and 3D noise reduction algorithm, 10 kinds of pseudo colors, capable of fire point detection, high temperature detection, and detection of regional intrusion, crossing the boundary, and entering/leaving the area. It supports 1 line temperature measurement, 9 frame temperature measurement, 10 point temperature measurement, 2 irregular areas, 2 circular area temperature measurement, full screen highest temperature, lowest temperature positioning, etc. It supports 2 alarm inputs and 2 alarm outputs. It is used in industrial places such as the power industry, chemical disposal centers, combustible warehouses, petrochemical plants, coal mines, etc., where real-time temperature monitoring of key locations is required to prevent dangerous events.

● Excellent image quality

● Intelligent detection

● Accurate temperature measurement

● Abnormal alarm

● Development interface: SDK development kit is provided

● Access standard: general version

Characteristic

Performance Indicators

| Product model | BW-DC13P |
| --- | --- |
| Thermal Imaging |  |
| Detector Type | Vanadium Oxide Uncooled Focal Plane Detector |
| Image Resolution | 384*288 |
| Element Size | 17μm |
| Detector NETD | ≦50mK@25℃ |
| Response Band | 8 ~14μm |
| Package Type | Ceramic Package |
| Visible light |  |
| Sensor | 1/2.8”CMOS |
| Graphic Resolution | 2592× 1536 |
| Minimum Illumination | 0.002Lux/F1.5(Colour) , 0.0002Lux/F1.5(B/W) |
| Infrared fill light | Infrared fill light 30 metres |
| Digital zoom | Support |
| Automatic Gain Control | ICR Infrared Filter |
| Image handling |  |
| Digital Video | H264 |
| Master Streaming Image | Visible light： 25fps(2592× 1536,1920× 1080,1280×720) |
| Start-up time | 10s |
| Image Processing | Non-uniformity correction, intelligent gain control, digital detail mixing enhancement, 3D noise reduction |
| Colour palette | Support 10 colour palettes such as Black Hot, White Hot, Iron Red, Rainbow, etc./Customizable |
| Image Mirroring | Left, right, up, down, centre |
| Function |  |
| Abnormal temperature alarm | Support temperature abnormality alarm |
| Temperature measurement function | Full-screen maximum and minimum temperature positioning |
|  | Support 1 line, 9 frames, 10 points, 2 irregular areas, 2 elliptical areas temperature measurement |
|  | Temperature range: -20℃~150℃ (extendable to 550℃)@-10~50℃. |
|  | Temperature measurement accuracy: ±2℃ or ±2% (take the greater value) no black body |
|  | Support reflective temperature, atmospheric temperature setting, target distance correction, support emissivity correction |
| Alarm | IP conflict, linkage alarm function |
| Internet |  |
| Ethernet Types | 100Mbps |
| Network Protocols | IPV4, HTTP, FTP, RTSP, RTCP, RTP, TCP, UDP |
| Concurrent Access | 20 way |
| Security Mode | Authorised users and passwords |
| Access Standard | Private SDK, ONVIF, GB/T28181 |
| Data interface |  |
| Compressed Data | H264 video, standard JPEG images |
| System Integration |  |
| Communication Interfaces | RJ45, Network Transmission |
|  | One RS485 |
| PTZ Control | Pelco-D protocol, RS-485 interface |
| Alarm Input/Output | 2 in/2 out |
| Power supply |  |
| Power Supply Range | DC12V |
| Typical Power Consumption | 2.2W |
| Physical characteristics |  |
| Product Size | 96mm*108mm*232mm |
| Weight (without accessories) | 845.5g(without stand) |
| Waterproof rating | IP66 |
| Environmental adaptability |  |
| Working Temperature/°C | -30℃～+60℃ |
| Storage Temperature/°C | -45℃～+75℃ |
| Humidity/RH | ≤95% |

Wuxi Bewis Sensing Technology LLC

Add： Building 30, NO. 58, Xi uxi Road, Binh u District,

Wuxi City, Jiangsu Province, China

Tel： +89 18921292620

Mail ：sales@bwsensing.com

Web：www.bwsensing.com

BW-DC13P series

Thermal Imaging Dual Spectrum Camera