Eleven-axis High-range Integrated Navigation System

BW-GI420

Technical Manual

V1.0

Introduction

GI420 is a cost-effective, long-range 11-axis integrated navigation product designed to provide customers with stable and reliable inertial measurement and navigation information. It undergoes rigorous calibration and compensation at the factory to ensure stable and reliable IMU data. In integrated navigation calculations, a high-performance fusion algorithm is used to deeply integrate IMU and GNSS data, resulting in high reliability and robustness.

GI420 supports IMU raw data output (three-axis gyroscope, three-axis accelerometer, three-axis magnetometer, barometric altimeter), GNSS position and velocity raw data output , and also supports integrated navigation information output , offering diverse options to meet customer needs. Additionally, it provides precise satellite time synchronization, providing a time reference.

It can meet the needs of customers in multiple scenarios, with applications including vehicles, ships, airplanes, gimbals, pods, etc. The GI420 integrated navigation algorithm can be customized and adapted to different dynamic models according to customer scenario requirements to achieve high-precision measurement and navigation positioning.

Features

High-precision integrated navigation		interface types are abundant

High performance, cost-effective			Supports full-system, full-frequency RTK

High-frequency, stable output			Rigorous calibration and compensation

Motion model adaptation supported		Supports AHRS

Supports external odometer              Supports customization

Supports raw 11-axis IMU data output

Wide range options available, suitable for high dynamics

Technical

IMUPerformance Parameters

| gyroscope | range | ±2000°/s | Maximum measurement ±2600°/s |
| --- | --- | --- | --- |
|  |  | ±300°/s |  |
|  | Zero bias stability | 6°/h | National military standard 10-second smoothing |
|  |  | 3°/h | National military standard 10-second smoothing |
|  | Zero bias instability | 2°/h | Allan variance |
|  |  | 0.5°/h | Allan variance |
|  | Zero-biased repeatability | <6°/h |  |
|  |  | <3°/h |  |
|  | Angle Random Walk | 0.26°/√h |  |
|  |  | 0.03°/√h |  |
|  | noise | 0.02°/s |  |
|  |  | 0.004°/s |  |
| accelerometer | range | ±16 g | Maximum measurement ±26g |
|  |  | ±8 g |  |
|  | Zero bias stability | 0.05 mg | National military standard 10-second smoothing |
|  |  | 0.08mg | National military standard 10-second smoothing |
|  | Zero bias instability | 0.015 mg | Allan variance |
|  |  | 0.022mg | Allan variance |
|  | Zero-biased repeatability | <0.3 mg |  |
|  |  | <0.35 mg |  |
|  | Speed Random Walk | 30 ( mm/s) /√h |  |
|  |  | 32 ( mm/s) /√h |  |
|  | noise | 0.008 m/s^2 |  |
|  |  | 0.00 9m/s^2 |  |
| magnetometer | Dynamic range (scale) | ±2Gs |  |
|  | Sensitivity | 15000 LSB/G |  |
|  | Field resolution | 2mGauss |  |
| barometric altimeter | Pressure range | 260 hPa~1260 hPa |  |
|  | resolution | 4096 LSB/hPa |  |
|  | accuracy | ±0.1 hPa | 25℃, 800-1100hPa |

Navigation performance parameters

| GNSS | satellite signal | BDS GPS Galileo GLONASS QZSS | B1I/B2I/B3I L1C/A/L2P(Y)/L2C/L5 E1/E5a/E5b G1/G2 L1/L2/L5 |
| --- | --- | --- | --- |
|  | Timing Accuracy (RMS) Velocity Accuracy (RMS) Speed Limit (RMS) | 20ns 0.03m/s 300m/s |  |
| GI420 Integrated Navigation | Roll Angle Range | ±180° |  |
|  | Pitch Angle Range | ±90° |  |
|  | Compass Range | 0~360° |  |
|  | Horizontal attitude accuracy (RMS) | 0.1° |  |
|  | Course Accuracy (RMS) | 0.1° |  |
|  | Course accuracy (GNSS lock loss) | <0.15° | Course drift (1 min) |
|  | RMS speed accuracy | 0.03m/s |  |
|  | Position accuracy (RMS, with accuracy limits determined by GNSS) | 1.5m 2.5m 0.4m 0.8m 0.8cm+1ppm 1.5cm+1ppm | SPP Horizontal Plane SPP Vertical DGPS Plane DGPS Altitude RTK Plane RTK Altitude |
|  | Ingress Protection Rating | IP67 |  |
|  | Dimensions | 46*45*32mm |  |
|  | Operating Voltage | DC 9~36V |  |
|  | Operating Temperature | -40~+85℃ |  |
|  | Storage Temperature | -55~+95℃ |  |

Interfaces

Interface Type

| GI420 device side | External terminals | / |
| --- | --- | --- |
| 12-pin metal plug (Female) | 12-pin metal socket (Male) | Power & Communication Interfaces |
| SMA (Female) | SMA male (Male) | GNSS main antenna interface |
| SMA (Female) | SMA male (Male) | GNSS from antenna interface |

Serial Port Settings

| Baud rate | 921600 / 460800 / 230400 / 115200 / 19200 / 9600 / 2400 |
| --- | --- |
| Data bits | 8 |
| Default configuration | 115200 8 1 No verification |
| Serial port optional | RS4 22 (RS4 85 ) / RS 232 |

Additional Interfaces

1-channel CAN FD interface

Packing List

| Type | Name | Model | Quantity | Remark |
| --- | --- | --- | --- | --- |
| Standard | 11-axis integrated navigation | GI420 | 1 | Net weight 65 g |
| Standard | Power & communication interface | 12-pin metal plug (male + 1m cable) | 1 | Net weight 63 g |
| Optional | GNSS antenna ① | Ceramic antenna 231 + SMA straight male feed cable 5m (integrated) | 2 |  |
| Optional | GNSS antenna ② | C03 mushroom-shaped antenna + 5m SMA direct- to-TNC feed cable | 2 |  |
| Optional | GNSS antenna ③ | Quad-arm spiral antenna + 5m SMA direct- to-SMA bus feeder | 2 |  |
| Optional | 4G DTU module | MD-649R | 1 |  |
| Optional | Serial port adapter module | DB9 (female) adapter board --- RS232/ RS422 | 1 |  |
| Optional | RS232 serial to USB | RS232 to USB --- DB9 (Male) | 1 |  |
| Optional | RS422 serial to USB | RS422 to USB --- DB9 (Male) | 1 |  |

Note:

GNSS antenna ①: Four-satellite full-band, dimensions 60*60*22.8mm, with magnet and 3M adhesive.

GNSS Antenna ②: Full-band, φ154mm, with studs and magnetic base.

GNSS Antenna ③: Four-satellite, multi-band, φ28*58.5mm.

Antenna ① is a ceramic-encapsulated antenna with 3M adhesive and a magnetic base. Its performance is slightly inferior to the quad-helix antenna ③ and the mushroom-shaped antenna ②. The mushroom-shaped antenna has the best performance but is the largest and heaviest. For airborne applications, the quad-helix antenna is recommended; for vehicle and shipborne applications, the mushroom-shaped antenna is recommended.

Pin Definition

| Serial Number | definition | describe | 12-pin aviation connector pins |
| --- | --- | --- | --- |
| 1 | A GND | negative terminal of power supply | PIN 1 |
| 2 | POWER_IN | DC_9-36V | Pin 2 |
| 3 | CANH | CAN signal pin | Pin 3 |
| 4 | CANL | CAN signal pin | Pin 4 |
| 5 | 232RXD | 232 communication pin | Pin 5 |
| 6 | 232TXD | 232 Communication pin | Pin 6 |
| 7 | DGND | Digital | Pin 7 |
| 8 | PPS_3.3V | Clock synchronization pulse (3.3V) | Pin 8 |
| 9 | 485_A/422_T+ | 485/422 communication pins | Pin 9 |
| 10 | 485_B/422_T- | 485/422 communication pin | Pin 10 |
| 11 | 422_R+ | 422 communication pin | Pin 11 |
| 12 | 422_R- | 422 communication pin | Pin 12 |

Slave antenna antenna

Main antenna

Hardware Connection Diagram

Installation

Dimensions: L46 × W45 × H32 (mm)

Dimensions

GNSS Antenna Specifications

The GNSS antenna used with the GI420 must be an active antenna. The GI420 requires a 5V DC antenna feed and supports a maximum current of 200mA. Recommended or required parameters are as follows.

- Frequency points to be supported：

| GNSS | frequency | GNSS | frequency |
| --- | --- | --- | --- |
| BDS | B1I/B2I/B3I | Galileo | E1/E5a/E5b |
| GPS | L1/L2/L5 | QZSS | L1/L2/L5 |
| GLONASS | G1/G2 | - | L-band |

- Recommended gain: 40dB;

- Recommended noise figure: NF < 1.5;

- Power supply: 2.8~5V;

- Phase center error: ±2mm.

RF Coaxial Cable Specifications

The main antenna RF coaxial cable must be impedance matched to both the antenna and receiver, with a characteristic impedance of 50Ω and a recommended line attenuation of < 10dB. One end of the RF coaxial cable connector is adapted to the GNSS antenna, and the other end is an SMA male connector adapted to the GI420 main antenna interface .

The antenna is the same as above, and the other end is an SMA male adapter for the GI420 antenna interface.

Temperature and IP Rating

The temperature requirements for GI420 are as follows:

1.Operating temperature : -40℃~+85℃

2.Storage temperature : -55℃ to +95℃

3.Protection rating: IP67

Power Supply Specifications

The GI420 has the following power supply requirements:

1.Voltage range 9~36V DC

2.At least 10W stable output power

GNSS Antenna Installation

The following precautions should be taken when installing a GNSS antenna:

1.The area above the GNSS antenna is open and unobstructed.

2.The GNSS antenna is rigidly connected to the carrier, ensuring that the antenna will not sway when the carrier moves.

3.For dual antennas, it is recommended that the antennas be as far apart as possible.

To simplify configuration, in GNSS dual-antenna mode, it is recommended that the baselines of both antennas be horizontal to the vehicle's forward direction, meaning the direction the main antenna points towards the secondary antenna should be consistent with the direction the rear of the vehicle points towards the front (main antenna at the rear, secondary antenna at the front) , as shown in the diagram below:

GI420 Installation

To improve accuracy, the IMU should be positioned as close as possible to the GNSS main antenna in the horizontal direction during installation . A rigid connection between the GI420 and the carrier must be ensured, guaranteeing that their relative positions on the carrier remain constant. Furthermore, the GI420 must be securely and reliably installed, preventing movement or shaking during carrier movement.

Installation Requirements: The GI420 should be horizontally aligned with the carrier (i.e., the Z-axis should be perpendicular to the carrier's upward direction). The Y-axis of the GI420 should point in the carrier's forward direction. It is essential to ensure that the Y-axis of the GI420, the direction from the main antenna to the secondary antenna, and the direction of the carrier's forward direction all point in the same direction (as shown in the diagram below) ：

Communication Connection

GI420 can communicate with external communication devices via serial port. Currently, commands and RTK data can only be sent through the RS232 port.

Serial Interface

GI420 integrated navigation system provides two serial ports, as shown in the table below :

| Serial number | RS-232 | RS-485 | RS-422 |
| --- | --- | --- | --- |
| COM1 | support | Not supported | Not supported |
| COM2 | Not supported | support | support |

Power Connection

The 12-core cable has pin 2 POWER_IN connected to the positive power supply and pin 1 AGND connected to the negative power supply.

Check GI420 Status

After installing the GI420, connect the power and send the command UNLOG. Note that when using a serial port assistant to send commands, you need to check "Send Newline" to confirm whether the GI420 is running normally. If so, the GI420 will reply with the following content:

$Command UNLOG response: OK。

Operation

Before use, ensure that the GI420 has been installed as described in the previous section and is powered on.。

Serial Communication

The GI420 can communicate with devices such as computers via serial port. Before establishing communication, both the GI420 and the computer need to have their serial port parameters properly configured. The default serial port configuration for the GI420 is as follows:

1.115200bps

2.No parity bit

3.8-bit data bits

4.1 stop bit

5.No parity bit

Only COM1 supports port configuration using the CONFIG command.

Here's an example of modifying the COM1 serial port configuration：

| Command Header | Serial port devices | Serial port parameters | Parameter Description |
| --- | --- | --- | --- |
| CONFIG | COM1 COM2 | baud rate | Set the serial port baud rate |

| instruction | describe |
| --- | --- |
| config com1 115200 | Set the baud rate of COM1 to 115200. COM1 supports baud rates of 2400, 9600, 19200, 115200, 230400, and 460800. COM2 supports the following baud rates: 2400, 9600, 19200, 115200, 230400, 460800, and 921600. |

The command format is:

CONFIG [Serial Port Device Number] [Serial Port Attributes]

GNGGA GNSS

This command is used to set the output of the multi-system joint positioning results for the current serial port or a specified serial port. The output information includes the GNSS receiver's time and positioning-related data. The statement begins with GNGGA. Depending on the satellite system involved in the positioning, it may be GPGGA, BDGGA, GLGGA, or GAGGA. When only the GPS satellite system is involved in the positioning calculation, it is output in GPGGA format; when only the BDS satellite system is involved, it is output in BDGGA format; when only the GLONASS satellite system is involved, it is output in GLGGA format; when only the Galileo satellite system is involved, it is output in GLGGA format.

Output in GNGGA format. If two or more satellite systems are involved in the positioning calculation, the output will be in GNGGA format.

Simplified ASCII format:

GNGGA 1 Currently outputs 1Hz GNGGA information via serial port.

GNGGA COM2 1 outputs 1Hz GNGGA information at COM2.

Message output:

$GNGGA,025754.00,4004.74102107,N,11614.19532779,E,1,18,0.7,63.3224,M,-

9.7848,M,00,0000*58

GNGGA data structure :：

| ID | Fields | Data Description | symbol | Example |
| --- | --- | --- | --- | --- |
| 1 | $GNGGA | Log header |  | $GNGGA |
| 2 | UTC | The corresponding UTC time for the location hh/mm/ss.ss | hhmmss.ss | 17 3568 .00 |
| 3 | lat | Latitude (DDmm.mm ) | IIII.II | 3251. 2654​ |
| 4 | Lat dir | Latitude direction (N = North, S = South) | a | N |
| 5 | lon | Longitude (DDDmm.mm ) | yyyyy.yy | 12033.3 59 2​ |
| 6 | lon dir | Longitude direction (E = East, W = West) | a | E |
| 7 | qual | GPS quality indicator 0 = Location unavailable or invalid 1 = Single point positioning 2 = Pseudorange differential or SBAS positioning 4 = RTK fixed solution 5 = RTK floating-point solution 6 = Inertial Navigation Positioning 7 = User-defined location ( Fixed) Position ) | x | 1 |
| 8 | # sats | Number of satellites in use. This may differ from the number seen. Inconsistent | xx | 10 |
| 9 | hdop | Horizontal precision factor | xx | 1.0 |
| 10 | alt | The antenna's altitude is below the geoid. It is a negative value. | xx | 10 21 .4 5 |
| 11 | a-units | Antenna height unit (M = m ) | M | M |
| 12 | undulation | Geoid Disparity – Geoid and the WGS84 ellipsoids. Large A positive value indicates the geoid is above the ellipsoid; otherwise... Then, the value is negative. | xx | -1 7.1 83​ |
| 13 | u-units | Geoid difference unit (M = m ) | M | M |
| 14 | age | Differential data age, in seconds | xx | 00 when there is no difference data ) |
| 15 | stn ID | Differential base station ID , 0000-4096 | xxxx | 00 when there is no difference data ) |
| 16 | *xx | Checksum | *hh | *3F |
| 17 | [CR][LF] | Statement terminator |  | [CR][LF] |

IMU raw data information

This statement contains IMU status indications and measurements from the accelerometer and gyroscope relative to the IMU housing coordinate system.

Command format: RAWIMUA COM1 1

RAWIMU data format:

RAWIMU data format:：

| ID | Fields | Data Description |
| --- | --- | --- |
| 1 | $RAWIMUA | Log header |
| 2 | Week | GNSS Week |
| 3 | Seconds into Week | Zhou Miao |
| 4 | X Accel Output | Acceleration along X Axis variation |
| 5 | Y Accel Output | Acceleration varies along the Y-axis |
| 6 | Z Accel Output | Acceleration varies along the Z-axis |
| 7 | X Gyro Output | Angular change of the right-hand screw along the X-axis |
| 8 | Y Gyro Output | Angular change of the right-hand screw along the Y- axis |
| 9 | Z Gyro Output | Along Z angular change of the right-hand screw of the shaft |
| 10 | X Mag Output | Magnetometer along X Axis variation |
| 11 | Y Mag Output | Magnetometer along Y Axis variation |
| 12 | Z Mag Output | Magnetometer along Z Axis variation |
| 13 | Barometer Output | barometer changes |
| 14 | *xx | Checksum |
| 15 | [CR][LF] | Statement terminator |

INSPVA Integrated Navigation Position, Velocity, and Attitude Information

To set the output of the combined navigation positioning result, ASCII statements must begin with "#INSPVAA".

Recommended input: INSPVAA com2 1

INSPVAA data structure：

| ID | Fields | Data Description |
| --- | --- | --- |
| 1 | $ INSPVA A | Log header |
| 2 | Week | GNSS Week |
| 3 | Seconds | Zhou Miao |
| 4 | Longitude | Longitude (WGS84) [degrees] |
| 5 | Latitude | Latitude (WGS84) [degrees] |
| 6 | Height | Ellipsoid height (WGS84) [m] |
| 7 | North Velocity | Northbound speed ( negative values for southbound ) [m/s] |
| 8 | East Velocity | Eastbound velocity ( negative values for westbound ) [m/s] |
| 9 | Down Velocity | Ground velocity [m/s] |
| 10 | Roll | Roll angle ( right-hand spiral along the Y-axis, counter-clockwise is positive) [ degrees ] |
| 11 | Pitch | Pitch angle ( right-hand screw along the X-axis, counter-clockwise is positive) [ degrees ] |
| 12 | Azimuth | The heading angle is 0° or 360° for true north ( northeast is positive). This is the inertial azimuth angle calculated by the IMU gyroscope through a combined filter. |
| 13 | Ahrs_Roll | The roll angle output by Ahrs ( right-hand spiral along the Y- axis, counter-clockwise is positive) [ degrees ] |
| 14 | Ahrs_Pitch | output by Ahrs ( right-hand screw along the X- axis, counter-clockwise is positive) [ degrees ] |
| 15 | Ahrs_Azimuth | The heading angle output by Ahrs is 0° or 360° for true north ( northeast is positive). This heading angle output is compensated for magnetic declination, therefore 0° or 360° is true north . |
| 16 | Track_Angle | Track angle, 0° or 360° for true north (northeast is positive) [ degrees ] |
| 17 | H_e | Height (in meters) |
| 18 | H_brao | Barometer height [meters] |
| 19 | Fbb_0 | The acceleration [g] of the carrier in the X direction, in units of g, is 9.794362833435068. |
| 20 | Fbb_1 | The acceleration [g] of the carrier in the Y direction, in units of g, is 9.794362833435068. |
| twenty one | Fbb_2 | The acceleration [g] of the carrier in the Z direction, in units of g, is 9.794362833435068. |
| twenty two | vel_body_0 | Carrier velocity in the X direction [m/s] |
| twenty three | vel_body_1 | Carrier velocity in the Y direction [m/s] |
| twenty four | vel_body_2 | Carrier velocity in the Z direction [m/s] |
| 25 | Gnssvalue | The Gnss valid flag is set to 0 for invalid and 1 for valid. |
| 26 | Sats | number of satellites |
| 27 | PositionStatus | Location status |
| 28 | DirectionStatus | Orientation state |
| 29 | Temperature | Temperature. Degrees Celsius (°C) |
| 30 | PDOP | Position accuracy factor |
| 31 | * xx | Checksum |
| 32 | [CR][LF] | Statement terminator ( ASCII only ) |

INSPVAB integrated navigation position, velocity, and attitude information

To set the output of the integrated navigation positioning result, the binary statement must begin with "#INSPVAB".

Recommended input: INSPVAB com2 1

INSPVAB data structure：

| ID | Fields |  | Data Description | type | byte count | Byte offset |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Synchronization segment |  | 0x57 | char | 1 | 0 |
| 2 | Frame information |  | 0x00 | char | 1 | 1 |
| 3 | Length segment |  | 0xcf | char | 1 | 2 |
| 4 | address range |  | 0x00 | char | 1 | 3 |
| 5 | command segment |  | 0x03 | char | 1 | 4 |
| 6 | Data segment | Week | GNSS Week | Ulong | 4 | 8 |
|  |  | Seconds | Zhou Miao | Double | 8 | 16 |
|  |  | Longitude | Longitude (WGS84) [degrees] | Double | 8 | twenty four |
|  |  | Latitude | Latitude (WGS84) [degrees] | Double | 8 | 32 |
|  |  | Height | Ellipsoid height (WGS84) [m] | Double | 8 | 40 |
|  |  | North Velocity | Northbound speed ( negative values for southbound ) [m/s] | Double | 8 | 48 |
|  |  | East Velocity | Eastbound velocity ( negative values for westbound ) [m/s] | Double | 8 | 56 |
|  |  | Down Velocity | Ground velocity [m/s] | Double | 8 | 64 |
|  |  | Roll | Roll angle ( right-hand helix along the Y- axis, counter-clockwise is positive) [ degrees ] | Double | 8 | 72 |
|  |  | Pitch | Pitch angle ( right-hand screw along the X- axis, counter-clockwise is positive) [ degrees ] | Double | 8 | 80 |
|  |  | Azimuth | The heading angle, rotated counterclockwise from north ( right-handed spiral around the Z-axis, with north-northeast as positive), is the inertial azimuth angle calculated by the IMU gyroscope through a combined filter. | Double | 8 | 88 |
|  |  | Ahrs_Roll | The roll angle output by Ahrs ( right-hand spiral along the Y- axis, counter-clockwise is positive) [ degrees ] | Double | 8 | 96 |
|  |  | Ahrs_Pitch | output by Ahrs ( right-hand screw along the X- axis, counter-clockwise is positive) [ degrees ] | Double | 8 | 104 |
|  |  | Ahrs_Azimuth | The heading angle output by Ahrs is 0° or 360° for true north ( northeast is positive). This heading angle output is compensated for magnetic declination, therefore 0° or 360° is true north . | Double | 8 | 112 |
|  |  | Track_Angle | Track angle, 0° or 360° for true north (northeast is positive) [ degrees ] | Double | 8 | 120 |
|  |  | H_e | Height (in meters) | Double | 8 | 128 |
|  |  | H_brao | Barometer height [meters] | Double | 8 | 136 |
|  |  | Fbb_0 | The acceleration [g] of the carrier in the X direction, in units of g, is 9.794362833435068. | Double | 8 | 144 |
|  |  | Fbb_1 | The acceleration [g] of the carrier in the Y direction, in units of g, is 9.794362833435068. | Double | 8 | 152 |
|  |  | Fbb_2 | The acceleration [g] of the carrier in the Z direction, in units of g, is 9.794362833435068. | Double | 8 | 160 |
|  |  | vel_body_0 | Carrier velocity in the X direction [m/s] | Double | 8 | 168 |
|  |  | vel_body_1 | Carrier velocity in the Y direction [m/s] | Double | 8 | 176 |
|  |  | vel_body_2 | Carrier velocity in the Z direction [m/s] | Double | 8 | 184 |
|  |  | Gnssvalue | The Gnss valid flag is set to 0 for invalid and 1 for valid. | char | 1 | 185 |
|  |  | Sats | number of satellites | char | 1 | 186 |
|  |  | PositionStatus | Location status | char | 1 | 187 |
|  |  | DirectionStatus | Orientation state | char | 1 | 188 |
|  |  | Temperature | Temperature. Degrees Celsius (°C) | Double | 8 | 196 |
|  |  | PDOP | Position accuracy factor | Double | 8 | 204 |
|  |  | Status | INS status | Enum | 1 | 205 |
| 7 |  | xxxx | 32-bit CRC | Hex | 4 | 209 |

Other instructions

Unlog stops serial port output

This command is used to stop the serial port from outputting specific data. The configurable parameter [statement] stops the output of the corresponding data; the configurable parameter [port] stops port output. If no port is specified, the default is the port currently receiving the command; if no message name is specified, all information output will be stopped.

The command format is:

UNLOG [port] [message]

Simplified ASCII syntax

UNLOG stops all output from the current serial port.

The UNLOG GNGGA statement stops the current serial port from outputting GNGGA.

UNLOG COM1 Stops all information output from COM1.

The UNLOG COM2 GNGGA statement stops the GNGGA output from COM2.

The Unlog command parameters are as follows：

| Command Header | port number | describe |
| --- | --- | --- |
| UNLOG | COM1 COM2 | The name of the message to stop output |

Saveconfig saves user configuration.

This command saves the current user configuration.

The command format is:

SAVECONFIG

Simplified ASCII syntax:

SAVECONFIG

Saveconfig command parameters are as follows：

| Command Header | Command parameters | describe |
| --- | --- | --- |
| SAVECONFIG | —— | Save user configuration |

IMU to main antenna mast parameter configuration

Use this command to input the offset between the IMU and the GNSS main antenna phase center, i.e., the inertial navigation system (INS) to main antenna boom parameter. The boom parameter should be measured as accurately as possible, especially in RTK mode; the error should ideally be within 1 cm. Any error in the boom parameter will be directly converted into an error in the INS position. X, Y, and Z represent the vector from the IMU to the main antenna phase center (measured with the IMU center as the origin and the coordinate system as right-front-upper) . To improve accuracy, the IMU should be installed as close as possible to the GNSS main antenna in the horizontal direction.

Offset from IMU to antenna phase center

Command format: CONFIG LEVER ARM xyz

Simplified ASCII syntax: CONFIG LEVER ARM 0.05 -1.05 0.03

IMU to main antenna boom parameter configuration

| Command Header | parameter | Parameter Description |
| --- | --- | --- |
| CONFIG LEVER ARM | X | X-direction offset, unit: meters, range: -100 to 100 , rounded to two decimal places. |
|  | Y | Y-direction offset, unit: meters, range: -100 to 100 , rounded to two decimal places. |
|  | Z | Z-direction offset, unit: meters, range: -100 to 100 , rounded to two decimal places. |

Command Set Table

| Order | Example | Notes and Explanations |
| --- | --- | --- |
| UNLOG COM1 |  | Turn off all outputs of 232. |
| UNLOG COM2 |  | Turn off all outputs of 485/422 |
| GNGGA COM1 1 |  | The RS-232 outputs GNGGA data at 1 Hz. When using RTK, the RS-232 needs to output GNGGA data to the Qianxun base station. |
| INSPVAA COM1 XX | INSPVAA COM1 0.05 | XX represents the reciprocal of the output frequency; in the example, it means that the 232 outputs data in INSVPA format at 20 Hz. |
| INSVAB COM1 XX | INSPVAB COM1 0.05 | XX represents the reciprocal of the output frequency; in the example, it means that the 232 outputs data in INSVAB format at 20Hz. |
| INSPVAA COM2 XX | INSPVAA COM2 0.05 | XX represents the reciprocal of the output frequency; in the example, it indicates that the 485/422 outputs data in INSPVA format at 20Hz. |
| INSPVAB COM2 XX | INSPVAB COM2 0.05 | XX represents the reciprocal of the output frequency; in the example, it indicates that the 485/422 outputs INSVAB format data at 20Hz. |
| CONFIG COM1 XXX | CONFIG COM1 460800 | XXX represents the baud rate. In this example, the baud rate of the RS-232 is set to 460800. RS-232 supports baud rates of 2400, 9600, 19200, 115200, 230400, and 460800. |
| CONFIG COM2 XXX | CONFIG COM2 460800 | XXX represents the baud rate. In the example, setting the baud rate for 485/422 indicates a baud rate of 460800. Supported baud rates for 232 are 2400, 9600, 19200, 115200, 230400, 460800, and 921600. |
| CONFIG LEVER ARM XX YY ZZ | CONFIG LEVER ARM 0.35 0.55 1.15 | XX YY ZZ represents the XYZ axis distance between the center of the main antenna and the center of the integrated navigation system, with the right front upper direction as the positive direction. The measurement of the boom arm is referenced in the previous section " IMU to Main Antenna Parameter Configuration ". Unit: meters. |
| CONFIG HEADING COMP XX | CONFIG HEADING COMP 30 | XX represents the heading angle between the line connecting the master and slave antennas (pointing from the master antenna to the slave antenna) and the longitudinal axis of the carrier (pointing to the carrier's forward direction), in degrees. This item is only required when there is a significant angle between the two; otherwise, it is not necessary. When shown in the diagram below, the angle is set to positive; otherwise, it is negative. The unit is degrees, and the range is [-360°, +360°]. |
| CONFIG LEVER ARM OTHER XX YY ZZ | CONFIG LEVER ARM OTHER 0.15 -0.35 1.05 | Configure the output position anywhere. Using the center of the integrated navigation system as the origin and the upper right-front-right direction as the positive direction, measure the XYZ axis distances from the desired output position to the center of the integrated navigation system. Refer to the previous section " IMU to Main Antenna Parameter Configuration " for the coordinate system of the integrated navigation system. Unit: meters. |
| CONFIG CARRIER MOD XX | CONFIG CARRIER MOD 1 | Configure the algorithm mode of the integrated navigation system. Different algorithm modes are configured according to different carriers and application scenarios. 0 represents airborne mode, 1 represents vehicle mode, 2 represents shipborne mode, and 3 represents construction machinery mode (agricultural machinery, excavators, mining trucks, etc.). |
| CONFIG BW VALUE XX | CONFIG BW VALUE 1 | Configure whether to remove the initial zero bias. 0 means do not remove, 1 means remove, and the default is 1 (removing the initial zero bias will make the integrated navigation perform better when GNSS lock is lost). |

Magnetometer calibration

- via COM 1 RS232. Open the serial port and configure it to transmit via HEX and receive via HEX.

- Position the sensor with a pitch angle of 0°, a roll angle of 0°, and a yaw angle pointing due north. Use this direction as the reference direction. If you don't know where due north is, you can use any direction as the reference direction. Keep the sensor stationary and send the start calibration command 77 05 00 E4 01 EA. The sensor will return 77 05 00 E4 01 EA, initiating calibration mode. After calibration mode is initiated, it will automatically send 77 05 00 E5 XX YY , where XX is the calibration count. This value will iterate continuously during motion calibration and converge and begin to increase when stationary.

- Slowly rotate the sensor one revolution around the X-axis.

- Slowly rotate the sensor one revolution around the Y-axis.

- Slowly rotate the sensor one full revolution around the Z-axis to complete the rotation step.

- Place the sensor still and wait for the return command 77 05 00 E5 XX YY, where XX is greater than or equal to 0x0A. The calibration is then considered complete.

- Send the calibration end command 77 05 00 E4 02 EB. Calibration complete.

- To erase magnetometer calibration parameters, send the command. 77 04 00 10 14

Appendix: Binary Protocol Format

| Synchronization segment 1 Byte | Frame information 1 Byte | Length segment 1/2/4 Byte | address range 1/2/4 Byte | command segment 1/2/4 Byte | Data segment 0 ~ n Byte | Ending paragraph 4 Byte |
| --- | --- | --- | --- | --- | --- | --- |

Data Frame Description

| name | length | illustrate |
| --- | --- | --- |
| Synchronization segment | 1 Byte | Fixed at 0x57 , used for data frame synchronization. |
| Frame information | 1 Byte | Information representing data frames Bit 7 frame information check: This bit is 0 when the number of 1s in Bits 6 to 0 of the frame information is even , and 1 when the number of 1s is odd . Bit 6 is reserved and always equal to 0. Bit 5 : Data length of a 4- bit segment 0 0 : 1 byte 0 1 : 2 bytes 0 2 : 4 bytes 0 3 : Reserved Bit 3 : Data length of the 2- bit address segment 0 0 : 1 byte 0 1 : 2 bytes 0 2 : 4 bytes 0 3 : Reserved Bit 1 : 0 Data length of the command segment 0 0 : 1 byte 0 1 : 2 bytes 0 2 : 4 bytes 0 3 : Reserved |
| Length segment | 1/2/4 Byte | The number of bytes from the address segment (inclusive) to the end segment (inclusive), with the most significant byte first, is determined by bits 5 : 4 of the frame information. |
| address range | 1/2/4 Byte | is determined by the 3 : 2 bits of the frame information. The sensor will only respond if this address is equal to the sensor address or equal to 0. |
| command segment | 1/2/4 Byte | is determined by the 3 : 2 ratio of the frame information bits. The least significant bit determines the function of the data frame; a least significant bit of 0 indicates that the data has been sent to the sensor, while a least significant bit of 1 indicates that the data has been returned from the sensor. |
| Data segment | 0 ~ n Byte | The data information corresponding to the command segment is generally sensor readings or configuration parameters. |
| Ending paragraph | 4 Byte | The data frame's checksum information consists of the CRC checksum from the frame information (inclusive) to the data segment (inclusive), with the high byte first. The CRC information is as follows: Width: 3 2 bits Polynomial: 04C11DB7 Initial value: FFFFFFFF XOR result: 00000000 Input reverse: No Output inversion: No |

Appendix: Change Log

April 28, 2026 — V1.0 Initial Release

Wuxi Bewis Sensing Technology LLC

Add： Building 30, NO. 58, Xi uxi Road, Binhu District,

Wuxi City, Jiangsu Province, China

Tel： +89 18921292620

Mail ：sales@bwsensing.com

Web：www.bwsensing.com

Eleven-axis High-range Integrated Navigation

BW-GI420