Technical Manuals

Dual Antenna Intelligent Positioning Terminal

BW-GI100 Series

V1.3

BW-GI100 is an intelligent module based on GNSS high-precision positioning technology and IoT communication technology, adopting high-precision MEMS IMU with built-in automatic compensation and filtering algorithm. It adopts unique integrated design with built-in GNSS full-band antenna receiving module, 4G IoT communication module, high-precision GNSS positioning board, high-precision posturing module and OTA online upgrade function. The board adopts excellent anti-electromagnetic interference design, compact and portable, beautiful and generous, can be applied to a variety of complex scenes, can be widely used in directional measurement and other fields.

Introduction

Main Feature

●RTK accuracy: Plane 0.8cm + 1ppm ;

Elevation 1.5cm + 1ppm;

●PPP accuracy：Plane 25cm; Elevation25cm；

●Orientation accuracy (0.6m baseline)：

±0.5°(1σ)，±1°(2σ)

● Satellite tracking

● Petroleum geological exploration

● Optical rangefinders

● GPS-assisted navigation

● Interface support: TTL

● Main board size: L40*W80*H12.6 (mm)

● Power supply voltage: 5V DC

● Operating temperature: -40°C to +70°C

Application

● Cloud Platform Equipment

● Marine surveys

● Underwater navigation

● Mechanical controls

| RTK | Plane Elevation |  | 0.8cm+1ppm 1.5cm+1ppm |
| --- | --- | --- | --- |
| ppp | Plane Elevation Convergence time |  | 25cm 25cm 15-25min |
| First positioning time | cold start warm start |  | <30s <15s |
| Maximum data rate | GNSS raw observations GNSS RTK positioning GNSS/INS solving IMU raw data rate Positioning information output |  | 20HZ 20HZ 200HZ 200HZ 1HZ |
| Orientation accuracy 0.6m baseline | 1σ 2σ |  | ±0.5° ±1° |

| Connector | Motherboard Interface 1x5-bit, 2mm Pitch Pin Connector |
| --- | --- |
| 4G-RTK antenna interface | I-PEX 20279（MHF） |
| GNSS main antenna interface | SSMA Female connector |
| GNSS slave antenna | MCX Female connector |
| Mounting | Four M3 screws (unthreaded) |

| Power voltage(V) | 5V DC |
| --- | --- |
| Working Current (mA) | 30mA（40mA max） |
| Operating Temperature (℃) | -40℃~+70℃ |
| Storage Temperature (℃) | -55℃~+100℃ |

Mechanical Index

Performance Index

Electrical Index

Product Feature

Motherboard Dimensions

| Serial |  | Name |  | Quantity |
| --- | --- | --- | --- | --- |
| 1 |  | GNSS main antenna, SMA Female to SSMA Male Adaptor |  | 1 |
| 2 |  | GNSS slave antenna with SMA/MCX-KJ adapter |  | 1 |
| 3 |  | 4G antenna and adapter cable |  | 1 |
| 4 |  | SIM card |  | 1 |
| 5 |  | Signal output cable with pins VCC, GND, TX, RX |  | 1 |

- Installation of SIM, install the SIM card in the corresponding slot

- Installation of the conversion connector, the bare board is marked with master and slave antenna female connector, according to the interface type in accordance with the completion can be

- To install the antenna, simply connect the antenna to the adapter connector

- Install the 4G antenna and use the 4G antenna connection cable to connect the 4G antenna to the 4G connector on the bare board.

- Setting the baud rate

- CONFIG COM1 115200  	Set the baud rate to 115200

- Setting the output frequency（currently supports two output frequencies）

- INSPVAB COM1 0.05  	20HZ Output

- INSPVAB COM1 1      	1HZ Output

- Stop output

- UNLOG

- Save Settings

- SAVECONFIG   	After setting the corresponding configuration, you need to save the settings, otherwise revert to the original state after reboot.

- Set Qianxun CORS account

- $CFG,NRTIP,XXXXX,XXXXX* 		XXXXXis the corresponding account number and password

- $CFG,NRTIP, , *           	  		Delete the corresponding account and password

- 4G Card Related Enquiry

- $ZJCX,4G,SIM*                 	Query 4G SIM status

- $ZJCX,4G,NET*                 	Query 4G network status

- $ZJCX,4G,TRAFFIC*             	Query 4G signal status

- $ZJCX,4G,CSQ*                 	Check 4G signal strength

- $ZJCX,4G,SIM,CCID*           	Query SIM card number

Instruction Description

Instructions for use

Accessory List

Wuxi Bewis Sensing Technology LLC

Add：Building 30, NO. 58, Xiuxi Road, Binhu District,

Wuxi City, Jiangsu Province, China

Tel：+86 18921292620

Mail：sales@bwsensing.com

Web：www.bwsensing.com

Dual Antenna Intelligent

Positioning Terminal

BW-GI100 Serial